Loading Magneto Update on MC909X using Update Loader 

An update package file is a ZIP file which contains the binaries for updating the OS, PowerMicro, Monitor, Splash screen and Application partition.
Update Loader Files:

Pkgs.lst – This is a list of the files to update.  They can be .cab files, .sig files or .bin files.  This file can be edited in Notepad or any other text editor.
Note: To retain the existing ‘Application’ partition on the terminal, remove the ‘909Xw50LxxAPxxxxxx.udl.bin’ entry from the ‘pkgs.lst’ file.
Update Files – the .cab, .bin or .sig files that will be written to the terminal.  The Partition Table and IPL partition CANNOT be updated using Update Loader.  If you want to update a terminal OS that was built with the Symbol Certificate, you can only use files that were also built with the Symbol Certificates.   

CleanPS.bin – the bin file that will cause the terminal to format the Persistent Store.  This should be the last file listed in pkgs.lst

StartUpdLdr.exe – This is the application that will find the pkgs.lst file and start the update process.  

Steps to be followed for Image update:
1) Place the Update Files (including CleanPS.bin) and the pkgs.lst file in the root of the SD card or the ‘\Temp’ folder of the terminal.  

2) Place the terminal into a Single-slot cradle or 4-slot cradle with Power supplied. 
3) Click on StartUpdLdr.exe to start the Image update process.

4) Alternatively, to allow image update to continue without AC Power 

a. The Flag "AllowUpdateOnBattery" has to be set (AllowUpdateOnBattery = 1) in “ULDRSETTINGS.CFG”. “ULDRSETTINGS.CFG” file has to be present in the same folder along with the Update files and pkgs.lst. 

b. The battery level at which image update can be performed is configurable using “BatteryUpdateLevel” in “ULDRSETTINGS.CFG”. The minimum threshold of “BatteryUpdateLevel” is 70%. (“BatteryUpdateLevel” range is 70-100)

Note: 
· "ULDRSETTINGS.CFG" is not required if Image Update is being run on AC Power.
· Setting the flag "AllowUpdateOnBattery" to zero would not allow Image Update to continue. Update warning splash screen would be displayed.
· Image update can be performed at 70% battery level, only after the terminals are upgraded to OEM version 01.39.0000.

When the process is complete, the terminal will boot to the calibration screen, this may take a while.  The calibration may go very slowly for 30 seconds or so.

Alternative procedures for updating form an SD card:

1) Place the Update Files (including CleanPS.bin) and the pkgs.lst file in the root of the SD card

2) Cold Boot the MC909X terminal (Press the primary battery release on the mobile computer to partially eject the battery from the mobile computer. While the battery is partially released, simultaneously press and release the handle trigger and the Power button. Push the battery to fully re-insert it in the mobile computer)
3) Within 1 second of Cold Boot, press and release the handle trigger to activate the Image update from SD card.
